# Quantitative Evaluation of LoRa RF Resiliency, Detectability, and Geolocation

# Project Description

This evaluation examined LoRa performance across three primary areas: resiliency, detectability, and geolocation. Resiliency testing measured how the LoRa link performed as simulated range increased and as an interfering signal was introduced into the RF environment. Detectability testing evaluated whether recorded LoRa signals could be reliably distinguished from noise using empirical ROC, histogram, and slash-curve analysis across different SNR and integration-time conditions. Geolocation testing used Cross Ambiguity Function processing to compare how changes in LoRa spreading factor and bandwidth affected the resulting CAF map, accumulated response region, and apparent transmitter location. Together, these tests were intended to compare waveform behavior under controlled and repeatable RFN scenarios rather than produce absolute real-world performance predictions.

This evaluation paper is the delivery of a Capstone Project for the Wireless Embedded Systems, Masters of Advance Study Program at University California San Diego (UCSD) and is a collaboration with Naval Information Warfare Center, Pacific (NIWC Pac) Division 535. Equipment and facilities at NIWC Pac were leveraged to conduct the evaluation.

For detectability, the evaluation focused on how reliably LoRa transmissions could be detected when the recorded RF signal was degraded by added Gaussian noise. Captured RF recordings were processed in MATLAB, where noise was inserted across SNR levels from 0 dB down to -40 dB. The analysis generated ROC curves, slash curves, and signal-versus-noise histograms to determine the relationship between probability of detection, probability of false alarm, integration time, and SNR. This allowed detectability to be evaluated empirically by observing how the signal-plus-noise and noise-only distributions separated or overlapped under different test conditions.

For geolocation, the evaluation focused on how changes in LoRa spreading factor and bandwidth affected CAF-based location results. RF recordings were collected from four receive points in the RFN environment, producing six receiver-pair combinations for CAF processing. Each pairwise CAF result was accumulated into a combined geolocation surface over a 100,000 meter by 100,000 meter search area. Rather than treating the output only as a pass/fail location estimate, the test compared how each waveform configuration changed the clarity, strength, and ambiguity of the CAF response, allowing spreading factor and bandwidth effects to be evaluated relative to one another.

For resiliency, the evaluation focused on the ability of the LoRa link to maintain packet delivery and signal quality as conditions became more challenging. The radios were moved through scripted RFN scenarios that simulated increasing range, and packet success rate, RSSI, and SNR were recorded from the LoRa development boards. Baseline unjammed runs were compared against runs with an emulated interference source to determine whether the added signal caused measurable degradation in link performance. This provided a controlled method for comparing how the waveform responded to range-related loss and interference within the same repeatable scenario.

https://github.com/Brycycle/WES-CAPSTONE

# Test Objectives

The objective of this evaluation was to collect repeatable RF test data that could be used to understand LoRa waveform behavior in areas relevant to the Wireless Embedded Systems program, DoD evaluation interests, and NIWC Pacific test methods. The objectives were divided into two categories: measurable objectives that could be marked as accomplished or not accomplished, and interpretive objectives intended to guide analysis, research, and lessons learned.

## Measurable Test Objectives

The following objectives define specific test products, outputs, or results that can be evaluated as accomplished or not accomplished:

- Collect valid and repeatable metrics for each of the three evaluation areas: resiliency, detectability, and geolocation.
Metric: Accomplished if the project produces usable test data and plots for the planned LoRa spreading factor and bandwidth combinations, including range-based resiliency data, radiometric detection outputs, and CAF-based geolocation outputs.

- Compare waveform behavior across LoRa spreading factor and bandwidth configurations.
Metric: Accomplished if the collected results allow each tested SF/BW combination to be compared against the others across the three evaluation areas.

- Generate a relative LoRa waveform tradeoff table.
Metric: Accomplished if the paper summarizes each tested SF/BW combination across range, data rate, resiliency, detectability, and geolocation performance using relative or comparative rankings.

- Document limitations that affect interpretation of the test results.
Metric: Accomplished if the paper identifies limitations such as simplified RFN propagation, lack of multipath, lack of Doppler, non-representative jammer power, and the distinction between controlled comparison results and real-world performance predictions.

- Determine whether the results support further LoRa evaluation at NIWC Pacific.
Metric: Accomplished if the paper provides a recommendation on whether LoRa should be evaluated further using higher-fidelity DoD-relevant scenarios.

## Interpretive Research Objectives and Questions

The following objectives were used to guide analysis, interpretation, and lessons learned. These are not strictly pass/fail objectives, but questions intended to expand understanding of LoRa waveform behavior:

- Determine whether LoRa’s chirp spread spectrum modulation, which supports long-range and low-SNR communication, also provides measurable resiliency under degraded or interfered conditions.

- Evaluate whether LoRa waveform characteristics provide any potential value for low-detectability scenarios.

- Determine how spreading factor and bandwidth independently or jointly affect resiliency, detectability, and geolocation results.

- Assess whether higher spreading factor improves performance in all tested areas, or whether improvements in one area create disadvantages in another.

- Determine whether bandwidth changes create meaningful differences in range performance, detection performance, or CAF geolocation behavior.

- Evaluate whether the traditional IoT trade space of range and data rate is sufficient for DoD-relevant evaluation, or whether additional factors such as resiliency, detectability, and geolocation should be included.

- Identify which LoRa configurations appear most favorable for range, data rate, resiliency, detectability, and geolocation within the controlled RFN environment.

- Determine what the results suggest about the strengths and weaknesses of the RFN test methods used during this evaluation.

- Identify what additional testing would be needed to transition from controlled RFN comparison to more operationally representative evaluation.

- Provide feedback to NIWC Pacific based on methods, results, and lessons learned from the Wireless Embedded Systems program.

# Test Setup and Methodology

The test setup for this evaluation used NIWC Pacific Building 535 facilities and RF test infrastructure to provide a controlled environment for LoRa waveform evaluation. The setup combined physical RF isolation, calibrated RF paths, laboratory test equipment, software-based control tools, and custom-developed LoRa end nodes. This allowed the project to evaluate resiliency, detectability, and geolocation using repeatable test conditions rather than over-the-air field measurements. The test bed was designed to support controlled waveform comparison across selected LoRa spreading factor and bandwidth combinations.

## Test Bed and RF Environment

Testing was performed using NIWC Pacific 535 facilities and the available Radio Frequency Network test infrastructure. The RFN environment provided the controlled test bed used to emulate range, receiver locations, signal routing, and scenario-driven RF conditions. RF isolated enclosures were used to contain the LoRa development boards and prevent uncontrolled external RF coupling. This allowed the radios to operate through the RFN and connected test equipment rather than radiating directly into the surrounding environment.

The RF paths included cable infrastructure and inline attenuation used to support RFN calibration and signal-level control. This calibration infrastructure was important because the test results depended on repeatable and comparable RF conditions between waveform configurations. By routing the LoRa radios through controlled RF paths, the evaluation was able to compare changes in packet success rate, RSSI, SNR, detectability outputs, and CAF geolocation behavior under scripted conditions.

## Equipment and Measurement Tools

The physical test setup included RF isolated boxes, calibrated cable paths, inline attenuators, signal generators, spectrum analysis equipment, and RF recording equipment. Signal generators were used to inject controlled RF signals into the environment, including the emulated interference source used during resiliency testing. Spectrum analysis equipment was used to observe and verify RF signal behavior during setup and calibration. RF recorders were used to capture signal data for post-processing, including the recordings used for detectability and CAF-based geolocation analysis.

The use of laboratory RF equipment allowed the test to maintain controlled and repeatable input conditions. However, the equipment setup was not intended to fully reproduce real-world propagation effects. Instead, the test bed provided a controlled environment for comparing waveform configurations against one another under the same defined scenarios.

## Software and Processing Tools

LabVIEW virtual instruments were used as part of the test execution and equipment-control environment. These VIs supported scripted testing, RFN operation, data capture, and test coordination. LabVIEW is commonly used for automated measurement and instrument-control workflows, which made it suitable for coordinating RF test equipment and repeatable test runs. NI documentation describes RF measurement workflows that generate and acquire RF test signals using software-controlled instrumentation, which is consistent with the role of LabVIEW in this test setup.

MATLAB was used for post-test analysis and signal processing. The MATLAB processing tools generated ROC curves, slash curves, histograms, and CAF geolocation outputs from recorded RF data. MATLAB’s signal-processing tools support visualization, measurement, and comparison of signals in time, frequency, and time-frequency domains, which aligns with the analysis workflow used in this project. The MATLAB suites used in this project were expanded to support the specific LoRa resiliency, detectability, and geolocation analysis requirements.

## Heltec LoRa V3 Development Boards

The LoRa end nodes used in this evaluation were developed using Heltec WiFi LoRa 32 V3 boards. These boards integrate an ESP32-S3 microcontroller with an SX1262 LoRa radio, making them suitable as configurable LoRa development platforms for waveform testing. Heltec describes the WiFi LoRa 32 V3 as an ESP32-S3 and SX1262-based LoRa node with a USB-C interface and onboard RF design features.

Development work on the Heltec boards focused on configuring the LoRa waveform parameters required for the evaluation, including spreading factor and bandwidth. The boards were programmed to transmit and receive test packets, report link metrics, and support repeatable test execution across the selected waveform configurations. Board-reported metrics such as packet success, RSSI, and SNR were used during resiliency testing, while the transmitted LoRa waveforms were also captured by RF recording equipment for detectability and geolocation processing.

The Heltec boards served as the project’s representative embedded LoRa radios rather than as final operational hardware. Their purpose was to provide a flexible and programmable LoRa platform that could be configured repeatedly across waveform settings and integrated into the NIWC Pacific RFN test environment. This development effort allowed the evaluation to connect embedded-system implementation work with controlled RF testing and post-processing analysis.

The software developed for this project can be found in the project’s github repository.

## Assumptions and Limitations

- Assumptions made before testing.

- Known limitations in the setup.

- Sources of possible error.

- A common limitation across all test areas was that the RFN environment simulated horizon/range effects under controlled, repeatable conditions, but did not include ripple, multipath propagation, or Doppler effects. Therefore, the resulting numerical values should not be interpreted as direct real-world performance predictions. Instead, the results are intended to provide a controlled basis for comparing waveform performance against other waveforms under the same defined scenarios.

# Results

## Detectability

### Purpose

The purpose of the detectability test was to evaluate how easily the LoRa signal could be observed, identified, or distinguished from the surrounding RF environment. For this project, detectability refers to the ability to recognize the presence of a LoRa transmission using RF measurement tools or signal analysis techniques. This test was intended to determine whether LoRa transmissions were visible in the spectrum and whether signal characteristics such as spread factor or bandwidth had a significant impact on the detectability.

At NIWC and in data-controlled environment, these results can be compared against other waveforms which is where the results are most valuable. One comparison waveform has been provided but no details or associated information of this data is provided due to proprietary and Controlled Unclassified Information (CUI) concerns.

### Test Method and execution

Detectability was evaluated by processing recorded LoRa RF signals through a MATLAB-based detection analysis suite. The purpose of this processing was to determine how reliably the LoRa signal could be detected under controlled signal-to-noise ratio conditions and varying integration times. The recorded RF signal was used as the baseline signal input, and additive Gaussian noise was inserted into the captured signal to generate multiple degraded signal conditions. The MATLAB function varied the signal-to-noise ratio from 0 dB down to -40 dB in order to evaluate detectability as the signal became increasingly buried in noise.

The detection method used for this analysis was radiometry, an energy-based signal detection technique that determines whether a signal is present by integrating the received energy over a defined time window and comparing that value against a detection threshold. In this approach, the detector does not need to decode the LoRa packet or know the exact transmitted data; instead, it evaluates whether the measured energy is statistically more consistent with a signal-plus-noise condition than a noise-only condition.

For each SNR condition, the MATLAB suite generated empirical receiver operating characteristic curves. These ROC curves compared probability of detection against probability of false alarm for different detection thresholds. The MATLAB process performed threshold-based sweeps across the integrated signal data to determine how often the signal was correctly detected and how often noise-only regions were incorrectly classified as signal. This allowed the detection performance to be evaluated empirically rather than relying only on a theoretical model.

The MATLAB suite also produced histogram plots of the signal-plus-noise and noise-only measurement distributions. These histograms were used to visualize the separation between the two classes. When the signal-plus-noise and noise-only distributions were well separated, the detector had a higher probability of detection with a lower probability of false alarm. When the distributions overlapped, the probability of false alarm increased because some noise-only integration windows produced values similar to the signal-present case. The amount of overlap between these empirical probability density functions provided a visual indication of how difficult the signal was to detect at each SNR.

In addition to ROC curves, slash curves were generated to evaluate the integration time required to achieve selected probabilities of detection. The MATLAB suite produced these curves for target detection probabilities of 0.95, 0.85, and 0.75 while varying integration time. These plots showed the relationship between integration time and required SNR for a selected probability of detection and probability of false alarm. This allowed the test to evaluate whether longer integration times improved detectability at lower SNR values.

Because the analysis required repeated threshold sweeps across multiple SNR levels and integration times, the MATLAB processing was computationally intensive. To reduce processing time, the detection analysis was run in parallel across four GPUs. The ROC curves, slash curves, and histogram plots were therefore generated from repeated empirical trials performed by the MATLAB suite. The resulting plots were used to assess how detectable the LoRa waveform was under decreasing SNR and increasing noise conditions.

### Results

All Histogram, ROC, and Slash curve plots for each configuration of LoRa are found in Section 7.1. The results covered here are a look at only the SF 7 and BW of 125 kHz configuration and a summary table of all configurations.

The table below is a summary of SNR values needed to achieve 3 different probabilities of detection values with an integration time of 1ms with the technique used for this test. These were obtained from the Slash curves.

| Waveform | Spread Factor | Bandwidth | SNR for .95/.05 (Pd/Pfa) | SNR for .85/.15 (Pd/Pfa) | SNR for .75/.25 (Pd/Pfa) |
| --- | --- | --- | --- | --- | --- |
| LoRa | 7 | 10.4 kHz | -2.5 dB | -5 dB | -7 dB |
| LoRa | 7 | 20 kHz | -2.5 dB | -5 dB | -7 dB |
| LoRa | 7 | 125 kHz | -5 dB | -7 dB | -9 dB |
| LoRa | 7 | 500 kHz | -8 dB | -10 dB | -12 dB |
| LoRa | 12 | 125 kHz | -5 dB | -7 dB | -9 dB |
| LoRa | 12 | 500 kHz | -8 dB | -10 dB | -12 dB |
| Compared WF | N/A | N/A | -22 dB | -24 dB | -25 dB |

### Interpretation

Explain what the results mean.

- Was detection consistent?

- At what conditions did detection improve or fail?

- What factors affected detectability?

### Validity Notes

Explain whether the detectability results are reliable.
If not, explain what may have gone wrong.

## Resiliency

### Purpose

The purpose of the resiliency test was to evaluate how well the LoRa communication link maintained performance as range increased and as an interfering signal was introduced into the RF environment. For this project, resiliency refers to the ability of the LoRa system to continue delivering usable packet traffic under degraded operating conditions, including increased separation distance, reduced received signal quality, and intentional RF interference. The test was intended to determine how packet success rate, RSSI, and SNR changed as the radios were moved through the simulated scenario and to compare performance between jammed and unjammed conditions.

### Test Method and execution

Resiliency testing was conducted using a scripted, scenario-driven RF test environment. Two LoRa radios were moved through a simulated environment using scripts played into the Radio Frequency Network equipment. These scripts emulated changing radio separation and allowed the link distance to extend up to 60 nautical miles. During each scenario, the LoRa development boards reported link performance values, including packet acknowledgment success, RSSI, and SNR. These values were recorded and plotted against simulated range to evaluate how the link degraded as distance increased.

Initial dry runs were performed to identify the approximate range region where link performance began to fall off. After this falloff region was identified, the formal test runs were started at a selected range that allowed the degradation trend to be captured in more detail. This approach allowed the test to focus on the portion of the scenario where packet success rate, RSSI, and SNR began to show measurable changes rather than spending the majority of the test time in a region where the link was either fully reliable or fully failed.

The first set of measurements was collected under an unjammed baseline condition. In this condition, the radios were moved through the scripted range scenario without the additional interfering signal present. The baseline data provided the reference performance for packet success rate, RSSI, and SNR as a function of range. The test was then repeated with an injected jamming signal present in the RF environment. The jammed test condition used the same general scenario so that the resulting packet success, RSSI, and SNR plots could be compared directly against the unjammed baseline.

The jamming signal was emulated using a signal generator. For this test, real-world jamming threat parameters were not used. Instead, the test used an emulated, legally obtainable software-defined radio source, represented as a USRP X104 with an output power of 23 dBm. This provided a controlled interference source that could be inserted into the RFN environment for comparison testing. The intent was not to represent the full capability of real-world jamming systems, but to create a repeatable interference condition that allowed the LoRa link response to be measured.

The primary outputs from the resiliency test were range-based plots comparing jammed and unjammed performance. Packet success rate was used as the main indicator of link reliability, while RSSI and SNR were used to evaluate how received signal quality changed over the same range profile. Comparing these plots allowed the test to show whether the injected interference caused a measurable reduction in packet acknowledgments, received signal strength, or signal-to-noise ratio relative to the baseline condition.

### Results

Present the observed behavior under different test conditions.

### Interpretation

Explain how well the system performed under stress, interference, degradation, or changing conditions.

### Validity Notes

Explain whether the resiliency test was valid.
Discuss any issues that may have affected the results.

would explicitly say that the 23 dBm emulated source was useful for controlled comparison, but not representative of real-world jammer power levels or threat behavior. That makes the test sound honest and technically defensible.

## Geolocation

### Purpose

The purpose of the geolocation test was to compare how changes in LoRa spreading factor and bandwidth affected Cross Ambiguity Function geolocation results. Rather than only determining whether the transmitter could be located, this test evaluated how each waveform configuration influenced the CAF map, accumulated response region, and apparent location estimate. The goal was to identify whether changes in spreading factor or bandwidth produced a significant difference in the strength, clarity, or ambiguity of the CAF-based geolocation output.

The results were not intended to represent absolute real-world geolocation accuracy. Instead, the test provided a controlled and repeatable method for comparing waveform-to-waveform geolocation behavior under the same scripted RFN conditions.

J. J. Johnson, “Implementing the cross ambiguity function and generating geometry-specific signals,” master’s thesis, Dept. Elect. Eng., Naval Postgraduate School, Monterey, CA, USA, Sep. 2001.

### Test Method

Geolocation testing was performed using RF recordings collected from four receive points within the RFN environment. These four points represented separate physical receiver locations observing the same LoRa transmission. The receiver locations were defined in the test geometry, and each RF recording was processed as an input to the MATLAB-based Cross Ambiguity Function analysis suite.

The MATLAB CAF processing suite was developed from the method presented by Johnson in Implementing the Cross Ambiguity Function and Generating Geometry-Specific Signals and expanded for this project’s LoRa geolocation evaluation. The original reference implemented CAF processing to compare two sampled signals and estimate time and frequency differences between them. For this test, the approach was applied to LoRa RF recordings collected at multiple receive points.

Because a single CAF calculation compares two receive points at a time, the four receiver recordings produced six unique receiver-pair combinations. A separate CAF calculation was performed for each pair. The resulting pairwise CAF outputs were then accumulated to produce a combined geolocation surface. This cumulative CAF map was used to identify the region where the pairwise signal comparisons most strongly agreed on the likely transmitter position.

The resulting output was plotted over a 100,000 meter by 100,000 meter area. In the CAF map, the receiver locations were represented by the defined geometry points, and the accumulated CAF response was displayed across the search area. Higher response regions indicated locations where the combined pairwise CAF results were more consistent with the measured RF recordings. The estimated transmitter location was identified from the region of strongest combined response and compared against the known simulated transmitter location.

A common limitation of this test was that the RFN environment provided a controlled and repeatable geometry, but did not fully model all real-world propagation effects. Specifically, the test did not include multipath propagation, Doppler effects, or other environmental channel variations that would normally affect geolocation performance. Therefore, the numerical position result should not be interpreted as a direct real-world accuracy prediction. The purpose of the test was to provide a repeatable method for comparing waveform-to-waveform geolocation behavior under the same scripted conditions.

### Results

Present estimated locations, error distances, maps, plots, or comparison tables.

### Interpretation

Explain how accurate or useful the geolocation results were.

- Were estimates stable?

- How large was the error?

- What conditions improved or degraded accuracy?

### Validity Notes

Explain whether the geolocation results are valid.
Discuss possible causes of error such as geometry, timing, signal quality, synchronization, calibration, or environmental effects.

# Overall Summary

## Objectives Met

| Measurable Objective | Status | Notes |
| --- | --- | --- |
| Collect valid and repeatable metrics for resiliency, detectability, and geolocation | Partially Accomplished | Accomplished Detectability and Geolocation metrics. Validity of resiliency metrics is questionable and requires further, recursively improved testing. See appendix for all results. |
| Compare waveform behavior across SF/BW configurations | Accomplished | See appendix for all results of each configuration. |
| Generate a relative LoRa waveform tradeoff table | Partially Accomplished | Accomplished Detectability and Geolocation tradeoffs. Validity of resiliency metrics is questionable and therefore not accomplished here. Further details below. |
| Document test limitations | Accomplished | Further details outlined below. |
| Determine whether further LoRa evaluation is justified | Accomplished | Results show preliminary performance of detectability could be valuable when compared to on-paper resiliency and range performance. Further evaluation at NIWC using real scenarios and improved LoRa development software to obtain valid resiliency metrics could prove valuable. Additional types of detection methods could be evaluated. Additionally, a greater range of SF and BW configurations could be explored to test a wider range of data rate capabilities. |

### Tradeoff table summary

### Limitations Captured Summary

## Discussion / Interpretation

- Determine whether LoRa’s chirp spread spectrum modulation, which supports long-range and low-SNR communication, also provides measurable resiliency under degraded or interfered conditions.

- Evaluate whether LoRa waveform characteristics provide any potential value for low-detectability scenarios.

- Determine how spreading factor and bandwidth independently or jointly affect resiliency, detectability, and geolocation results.

- Assess whether higher spreading factor improves performance in all tested areas, or whether improvements in one area create disadvantages in another.

- Determine whether bandwidth changes create meaningful differences in range performance, detection performance, or CAF geolocation behavior.

- Evaluate whether the traditional IoT trade space of range and data rate is sufficient for DoD-relevant evaluation, or whether additional factors such as resiliency, detectability, and geolocation should be included.

- Identify which LoRa configurations appear most favorable for range, data rate, resiliency, detectability, and geolocation within the controlled RFN environment.

- Determine what the results suggest about the strengths and weaknesses of the RFN test methods used during this evaluation.

- Identify what additional testing would be needed to transition from controlled RFN comparison to more operationally representative evaluation.

- Provide feedback to NIWC Pacific based on methods, results, and lessons learned from the Wireless Embedded Systems program.

# Final Conclusions

| SF Value | BW | Frequency | Tx Power | Thermal noise estimate | SX1262 Rx sensitivity (spec-typ) | DR (theoretical, CR = 4/5, preamble on) |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | 10.4kHz | 951MHz | 22dBm - | -133.6 dBm | -134 dBm |  |
| 12 | 10.4kHz | 951MHz | 22dBm - | -133.6 dBm | -148 dBm |  |
| 7 | 125kHz | 951MHz | 22dBm - | -122.8 dBm | -124 dBm |  |
| 12 | 125kHz | 951MHz | 22dBm - | -122.8 dBm | -137 dBm |  |
| 7 | 500kHz | 951MHz | 22dBm - | -116.8 dBm | -117 dBm |  |
| 12 | 500kHz | 951MHz | 22dBm - | -116.8 dBm | -129 dBm |  |

| SF | BW (kHz) | Sym Rate (sym/s) | Time on Air (s) | LISTEN_WINDOW (ms) |
| --- | --- | --- | --- | --- |
| 7 | 10.4 | 81.25 | 1.2 | 2000 |
| 7 | 125 | 976.56 | 0.1 | 250 |
| 7 | 500 | 3906.25 | 0.025 | 250 |
| 12 | 10.4 | 2.54 | 39 | 90000 |
| 12 | 125 | 30.52 | 3.3 | 5000 |
| 12 | 500 | 122.07 | 0.82 | 1500 |

*note – 50byte payload

| SF | BW (kHz) | Sym Rate (sym/s) | Time on Air (s) | LISTEN_WINDOW (ms) |
| --- | --- | --- | --- | --- |
| 7 | 10.4 | 81.25 | 2.5 | 5000 |
| 7 | 125 | 976.56 | 0.21 | 500 |
| 7 | 500 | 3906.25 | 0.05 | 250 |
| 12 | 10.4 | 2.54 | 98 | 150000 |
| 12 | 125 | 30.52 | 8.2 | 15000 |
| 12 | 500 | 122.07 | 2 | 5000 |

*note – 125byte payload

SX1262 LoRa node chip

Max Tx 22dBm

Freq range 150MHz – 960MHz

BW range 7.8/10.4 – 500kHz

SF (chips/sym = 2^SF) = 5-12

Thermal noise power level estimate

10log10(delta f) – 173.8 dBm for BW in Hz.

# Appendix of Results

## Probability of detection

### SF 7 BW 10.4 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### SF 7 BW 20 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### SF 7 BW 125 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### SF 7 BW 500 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### SF 12 BW 125 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### SF 12 BW 500 kHz

#### Histograms

#### ROC Curves

#### Slash Curves

### Comparison Waveform

#### Histograms

#### ROC Curves

#### Slash Curves

## Geolocation

| SF 7 BW 10.4 kHz | SF 7 BW 20 kHz |
| --- | --- |
| SF 7 BW 125 kHz | SF 7 BW 500 kHz |
| SF 12 BW 125 kHz | SF 12 BW 500 kHz |

## Range and Resiliency

### SF 7 BW 10.4 kHz

### SF 7 BW 20 kHz

### SF 7 BW 125 kHz

### SF 7 BW 500 kHz

### SF 12 BW 125 kHz

### SF 12 BW 500 kHz